#include<standard_disclaimer>

--- ** THE CROSSOVER DESIGNER V1.5 ** ---

a program by Dan Cousin

compiled 1-27-96

on MSDOS 6.22

:DESCRIPTION

This is a little program I wrote to do quick and dirty crossover

crossover calculations for my loudspeaker building projects. The

formulas used in this version are from Vance Dickesons, "The

Loudspeaker Design Cookbook" (fourth edition) excluding a few

typographical errors I found.  It designs crossover networks based on

a resistive load.  As all experienced loudspeaker designers know

there are few drivers that come out of the box with a flat impedance

vs frequency plot so YOU must flatten the impedance curve in order

for these circuits to work properly.  I have included three utilities

to help you do just that.

:IMPEDANCE LEVELING CIRCUIT

Connecting the impedance leveling circuit does a good job of making

the impedance flatter but it is not perfect. The only way to get

it perfect is to run an impedance curve on the driver to see just

how flat the curve is, then make adjustments as required to get a

flat impedance. This circuit in used to counter the rise in impedance

due to the inductance of the voice coil.  For example, a speaker with

a resonant frequency of 40 Hz after this circuit is applied would have

a flatter impedance from say 200 Hz up to the upper limit of the driver.

This is useful for high pass crossovers on woofers in two-way systems,

but can be used for midrances and tweeters as well. Your amplifier is much

happier when it drives a load (SPEAKER) with a flat impedance.

:RESONANCE SMOOTHING RLC

The resonance smoothing notch filter options both try to do

the same thing which is reduce the impedance peek at the resonance

frequency of a driver.  You DO NOT need to worry about this circuit

if you have tweeters with ferofliud on the voice coil, because the

ferofluid does a good job flattening this peek mechanically.

Tweeters without ferofluid benifit greatly from these circuits

because the crossover is usually active over the area of the

resonance peek of the tweeter.  With this peek, don't think for a

second that your tweeter response curve is going to look even

close to what you would expect.

The option using the Q parameters is the more accurate of the two

so if you have these numbers use them.  Once again the given circuit

is not perfect and you should run an impedance curve to see how close

it got and make changes as neccessary to get a flat impedance.  This

circiut operates ONLY in a small range of frequencies around the

resonance frequency of the driver.

It is not recommended to use this circuit on bass drivers because

capacator and inductor values get very large.

:GENERAL DESIGN METHOD

1)  Apply impedance correction circuits to drivers.

Try to get a flat impedance over the ranges where you

plan to crossover the drivers

2)  Get an acoustic response curve for each driver to pick

good frequencies to place the crossover(s).

3)  Apply crossover network.

4)  Check acoustc response curve again to see if you got a flat

acoustic response over operating frequencies.

5)  if everything works out, ENJOY!

:DISCLAIMER

This software comes as-is.  The programmer assumes NO responsibility

for damages by, or from the use of this program. Use at your own risk.

Although I have used this program as a tool to build several projects

I can't guarantee it will work on every computer.  I have run this

program on a AMD 486dx/2-80 with an ati mach32,1 meg video card, a

Cyrix 486dlc40 with an OAK?? 256k video card, and an old standard

IBM P/S1 386-16 with standard VGA card.  This program does require

your card to provide 50 line text mode when drawing the circuits, so

this is the only hardware requirement I can think of.

:Shameless Begging

This software is Beg-Ware, meaning that I did not cripple

any of the functions to make you BUY the real version.  The

version you have is what I use for my own projects.  So PLEASE

if you find this software useful send me a token of your

appreciation $$$.

Dan Cousin

P.O. Box 22

Madisonville, La 70447

dcousin@tiger.lsu.edu

E-mail comments and questions to me. I would like your feedback.