Two common problems with this monitor are deflection faults resulting from failure of CXA2043Q, and video (colour) faults resulting from failure of FA4111 (colour will disappear after warmup - confirm IC is bad by using freeze spray ). Replacing the CXA2043Q requires above average soldering skills to not damage the main PCB during replacement.