BIPOLAR ANALOG INTEGRATED CIRCUITS

UPC2711TB, UPC2712TB

5 V, SUPER MINIMOLD SILICON MMIC WIDEBAND AMPLIFIER

## FEATURES

- HIGH DENSITY SURFACE MOUNTING:

6 pin super minimold or SOT- 363 package

- SUPPLY VOLTAGE: VCC = 4.5 to 5.5 V

- WIDEBAND RESPONSE: UPC2711TB: fu = 2.9 GHz TYP UPC2712TB: fu = 2.6 GHz TYP

- POWER GAIN:

UPC2711TB: GP = 13 dB TYP UPC2712TB: GP = 20 dB TYP

## DESCRIPTION

The  UPC2711TB  and  UPC2712TB  are  Silicon  MMIC Wideband Amplifiers manufactured using NEC's 20 GHz fT NESATTM  III silicon bipolar process.  These devices are de- signed for use as buffer amps in DBS tuners. The UPC2711/ 12TB are pin compatible and have comparable performance as the larger UPC2711/12T, so they are suitable for use as a replacement to help reduce system size.   These IC's are housed in a 6 pin super minimold or SOT-363 package.

Stringent quality assurance and test procedure ensure the highest reliability and performance.

## TYPICAL PERFORMANCE CURVES

#### GAIN vs. FREQUENCY

25

UPC2712TB

Gain, GP (dB)

20

15	UPC2711TB

10

5

0.5	1.0	1.5	2.0	2.5	3.0

Frequency, f (GHz)

ELECTRICAL CHARACTERISTICS (TA = +25 C, VCC = 5.0 V, ZL = ZS = 50 W)

| PART NUMBER PACKAGE OUTLINE |  |  | UPC2711TB S06 |  |  | UPC2712TB S06 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SYMBOLS | PARAMETERS AND CONDITIONS | UNITS | MIN | TYP | MAX | MIN | TYP | MAX |
| ICC | Circuit Current (no signal) | mA | 9 | 12 | 15 | 9 | 12 | 15 |
| GP | Power Gain, f = 1 GHz | dB | 11 | 13 | 16.5 | 18 | 20 | 23.5 |
| fU | Upper Limit Operating Frequency (The gain at fU is 3 dB down from the gain at 100 MHz) | GHz | 2.7 | 2.9 |  | 2.2 | 2.6 |  |
| ΔGP | Gain Flatness, f = 0.1 GHz to 2.5 GHz | dB |  | 0.8 |  |  | 0.8 |  |
| PO(SAT) | Maximum Output Level, f = 1 GHz, PIN = 0 dBm | dBm | -2 | +1 |  | 0 | +3 |  |
| NF | Noise Figure, f = 1 GHz | dB |  | 5 | 6.5 |  | 4.5 | 6 |
| RLIN | Input Return Loss, f = 1 GHz | dB | 20 | 25 |  | 9 | 12 |  |
| RLOUT | Output Return Loss, f = 1 GHz | dB | 9 | 12 |  | 10 | 13 |  |
| ISOL | Isolation, f = 1 GHz | dB | 25 | 30 |  | 28 | 33 |  |

The information in this document is subject to change without notice. Before using this document, please confirm that this is the latest version.

Date Published: June 22, 2005

ABSOLUTE MAXIMUM RATINGS1 (TA = 25C)

## RECOMMENDED OPERATING CONDITIONS

| SYMBOLS | PARAMETERS | UNITS | RATINGS |
| --- | --- | --- | --- |
| VCC | Supply Voltage | V | 6 |
| PIN | Input Power | dBm | +10 |
| PT | Total Power Dissipation2 | mW | 200 |
| TOP | Operating Temperature | C | -45 to +85 |
| TSTG | Storage Temperature | C | -55 to +150 |

| SYMBOL | PARAMETER | UNITS | MIN | TYP | MAX |
| --- | --- | --- | --- | --- | --- |
| VCC | Supply Voltage | V | 4.5 | 5.0 | 5.5 |
| TOP | Operating Temperature | ˚C | -40 | +25 | +85 |

Notes:

- Operation in excess of any one of these parameters may result in permanent damage.

- Mounted on double sided copper clad 50 x 50 x 1.6 mm epoxy glass PWB (TA = +85C).

## PIN DESCRIPTION

| Pin No. | Pin Name | Applied Voltage (V) | Description | Internal Equival | ent Circuit |
| --- | --- | --- | --- | --- | --- |
| 1 | Input |  | Signal input pin. An internal matching |  | 6 |
|  |  |  | circuit, configured with resistors, enables |  |  |
|  |  |  | 50 connection over a wide bandwidth. |  |  |
|  |  |  | A multi-feedback circuit is designed to |  |  |
|  |  |  | cancel the deviations of hFE and resistance. |  | 4 |
|  |  |  | This pin must be coupled to the signal source |  |  |
|  |  |  | with a blocking capacitor. |  |  |
| 4 | Output |  | Signal output pin. An internal matching circuit, configured with resistors, enables 50 connection over a wide bandwidth. This pin must be coupled to the output load with a blocking capacitor. | 1 |  |
| 6 | VCC | 4.5 to 5.5 | Power supply pin. This pin should be externally equipped with a bypass capacitor to minimize ground impedance. |  | 2 |
| 2 3 5 | GND | 0 | Ground pin. This pin should be connected to system ground with minimum inductance. Ground pattern on the board should be formed as wide as possible. All the ground pins must be connected together with wide ground pattern to minimize impedance difference. |  |  |
|  |  |  |  |  | 3 |
|  |  |  |  |  | 5 |

TYPICAL PERFORMANCE CURVES (TA = 25C)

#### UPC2711TB CURRENT vs. VOLTAGE

20

#### UPC2711TB CURRENT vs. TEMPERATURE

VCC = 5.0 V

20

18	18

16	16

Current, ICC (mA)

Current, ICC (mA)

14	14

12	12

10	10

8	8

6	6

4	4

2	2

0	1	2

3	4	5	6

0

-60

-40

-20

0	20	40	60	80    100

Voltage, VCC (V)

Temperature, TA (C)

#### UPC2711TB

GAIN AND NOISE FIGURE vs. FREQUENCY AND VOLTAGE

VCC = 5.5 V

VCC = 5.0 V

GS

VCC = 4.5 V

NF

VCC = 5.0 V

VCC = 4.5 V

VCC = 5.5 V

20

#### UPC2711TB

GAIN vs. FREQUENCY AND TEMPERATURE

TA = -40C

TA = +25C

TA = +25C

TA = -40C

VCC = 5.0 V

TA = +85C

TA = +25C

15

Noise Figure, NF (dB)

15	8

Gain, GS (dB)

Gain, GS (dB)

10	7

10

5	6

0	5

-5

0.1

0.3

4

1.0	3.0

5

0.1

0.3

1.0	3.0

Frequency, f (GHz)

Frequency, f (GHz)

0

Isolation, ISOL, (dB)

-10

-20

-30

-40

-50

|  |  |  |  |  |  |  |  |  | VCC = 5.0 V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

UPC2711TB ISOLATION vs. FREQUENCY

Input Return Loss, RLIN (dB) Output Return Loss, RLOUT (dB)

0

-10

-20

-30

-40

#### UPC2711TB

|  |  |  |  |  |  |  |  |  | VCC = 5.0 V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RLOUT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | RLIN |  |

INPUT RETURN LOSS AND OUTPUT RETURN LOSS vs. FREQUENCY

TYPICAL PERFORMANCE CURVES (TA = 25C)

#### UPC2711TB OUTPUT POWER vs.

INPUT POWER VOLTAGE

f = 1.0 GHz

5.5 V

VCC = 5.0 V

4.5 V

5

#### UPC2711TB OUTPUT POWER vs.

INPUT POWER AND TEMPERATURE

VCC = 5.0 V

f = 1.0 GHz

+85C

TA = +25C

-40C	-20C

5

Output Power, POUT (dBm)

0

-5

-10

-15

Output Power, POUT (dBm)

0

-5

-10

-15

-20

-35     -30    -25	-20     -15     -10	-5	0	5

Input Power, PIN (dBm)

-20

-35     -30    -25	-20     -15     -10	-5	0	5

Input Power, PIN (dBm)

#### UPC2711TB OUTPUT POWER vs.

INPUT POWER AND VOLTAGE

f = 2.0 GHz

5.5 V

VCC = 5.0 V

4.5 V

5

#### UPC2711TB OUTPUT POWER vs.

INPUT POWER AND FREQUENCY

VCC = 5.0 V

f = 0.5 GHz

f = 1.0 GHz

f = 2.0 GHz

f = 2.9 GHz

5

Output Power, POUT (dBm)

0

-5

-10

-15

Output Power, POUT (dBm)

0

-5

-10

-15

-20

-35     -30    -25	-20     -15     -10	-5	0	5

-20

-35     -30    -25	-20     -15     -10	-5	0	5

Input Power, PIN (dBm)

#### UPC2711TB SATURATED OUTPUT POWER vs.

FREQUENCY AND VOLTAGE

5.5 V

PIN = 0 dBm

VCC = 5.0 V

4.5 V

10

Input Power, PIN (dBm)

#### UPC2711TB

3RD ORDER INTERMODULATION DISTORTION vs. OUTPUT POWER AND VOLTAGE

VC

C = 5.0 V

4.5 V

5.5 V

f1 = 1.000 GHz

f2 = 1.002 GHz

0

Saturated Output Power, PSAT (dBm)

5

0

-5

-10

-10

3rd Order Intermodulation Distortion, IM3 (dBc)

-20

-30

-40

-15

0.1	0.3	1.0	3.0

-50

-20   -18   -16   -14   -12  -10    -8     -6     -4     -2	0

TYPICAL PERFORMANCE CURVES (TA = 25C)

#### UPC2712TB CURRENT vs. VOLTAGE

20

#### UPC2712TB CURRENT vs. TEMPERATURE

VCC = 5.0 V

20

18	18

16	16

Current, ICC (mA)

Current, ICC (mA)

14	14

12	12

10	10

8	8

6	6

4	4

2	2

0	1	2

3	4	5	6

0

-60

-40

-20

0	20	40	60	80    100

Voltage, VCC (V)

Temperature, TA (C)

#### UPC2712TB

GAIN AND NOISE FIGURE vs. FREQUENCY AND VOLTAGE

30

#### UPC2712TB GAIN vs.

FREQUENCY AND TEMPERATURE

VCC = 5.0 V

TA = +85C

-40C

+25C

25

25

VCC = 5.0 V

Gain, GS (dB)

GS

20

15	VCC = 4.5 V

NF

10

Noise Figure, NF (dB)

VCC = 5.5 V

Gain, GS (dB)

8

20

6

10

VCC = 5.5 V

4

VCC = 5.0 V

5

0.1

0.3

2

1.0	3.0

15

0.1

0.3

1.0	3.0

Frequency, f (GHz)

Frequency, f (GHz)

#### UPC2712TB ISOLATION vs. FREQUENCY

UPC2712TB

INPUT RETURN LOSS AND OUTPUT RETURN LOSS vs. FREQUENCY

VCC = 5.0 V

0

Isolation, ISOL, (dB)

-10

-20

-30

-40

-50

nput Return Loss, RLIN (dB) Output Return Loss, RLOUT (dB)

|  |  |  |  |  |  |  |  |  | VCC = 5.0 V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RLIN |  |  |  |  |  |  |  |  |  |  |
| RLOUT |  |  |  |  |  |  |  |  |  |  |

0

-10

-20

-30

-40

TYPICAL PERFORMANCE CURVES (TA = 25C)

#### UPC2712TB OUTPUT POWER vs.

INPUT POWER VOLTAGE

5.5 V

f = 1.0 GHz

VCC = 5.0 V

4.5 V

10

#### UPC2712TB OUTPUT POWER vs.

INPUT POWER AND TEMPERATURE

VCC = 5.0 V f = 1.0 GHz

+85C

TA = +25C

-40C

10

Output Power, POUT (dBm)

5

0

-5

-10

-15

-20

5

Output Power, POUT (dBm)

0

-5

-10

-15

-20

-25

-45  -40   -35   -30   -25   -20   -15   -10    -5	0     5

Input Power, PIN (dBm)

-25

-40     -35    -30    -25    -20    -15    -10     -5	0	5

Input Power, PIN (dBm)

#### UPC2712TB OUTPUT POWER vs.

INPUT POWER AND VOLTAGE

5.5 V

f = 2.0 GHz

VCC = 5.0 V

4.5 V

10

#### UPC2712TB OUTPUT POWER vs.

INPUT POWER AND FREQUENCY

f = 0.5 GHz

VCC = 5.0 V

f = 1.0 GHz

f = 2.0 GHz

10

Output Power, POUT (dBm)

5

0

-5

-10

-15

-20

5

Output Power, POUT (dBm)

0

-5

-10

-15

-20

-25

-40     -35    -30    -25    -20    -15    -10     -5	0	5

-25

-45  -40    -35   -30   -25   -20   -15   -10    -5	0     5

Input Power, PIN (dBm)

#### UPC2712TB SATURATED OUTPUT POWER vs.

FREQUENCY AND VOLTAGE

Input Power, PIN (dBm)

#### UPC2712TB

3RD ORDER INTERMODULATION DISTORTION vs. OUTPUT POWER AND VOLTAGE

5.5 V

PIN = 2 dBm

VCC = 5.0 V

4.5 V

f1 =

1.000 G

Hz

f2 =

1.002 G

Hz

VCC = 5.0 V

4.5 V

5.5 V

10	0

Saturated Output Power, PSAT (dBm)

3rd Order Intermodulation Distortion, IM3 (dBc)

-10

5

-20

0

-30

-5

-40

-10

0.1	0.3	1.0	3.0

-50

-16    -14	-12     -10	-8	-6	-4	-2	0

TYPICAL SCATTERING PARAMETERS (TA = 25C)

## UPC2711TB

VCC = 5 V, ICC = 13.8 mA

FREQUENCY	S11	S21	S12	S22	K

| (MHz) 100.00 | MAG 0.085 | ANG -22.40 | MAG 4.447 | ANG -14.90 | MAG 0.035 | ANG -12.70 | MAG 0.113 | ANG -3.10 | 3.18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200.00 | 0.086 | -25.00 | 4.468 | -30.10 | 0.035 | -23.00 | 0.119 | 1.20 | 3.21 |
| 300.00 | 0.098 | -29.20 | 4.491 | -44.90 | 0.034 | -32.10 | 0.136 | 1.60 | 3.23 |
| 400.00 | 0.081 | -29.40 | 4.510 | -60.30 | 0.033 | -42.50 | 0.142 | 6.50 | 3.34 |
| 500.00 | 0.066 | -33.90 | 4.540 | -74.90 | 0.033 | -50.10 | 0.156 | 10.10 | 3.32 |
| 600.00 | 0.041 | -54.50 | 4.572 | -90.20 | 0.033 | -59.60 | 0.161 | 12.70 | 3.34 |
| 700.00 | 0.053 | -104.30 | 4.624 | -105.30 | 0.032 | -69.30 | 0.161 | 8.80 | 3.33 |
| 800.00 | 0.070 | -119.70 | 4.664 | -120.70 | 0.031 | -78.40 | 0.176 | 6.20 | 3.36 |
| 900.00 | 0.098 | -121.90 | 4.729 | -136.10 | 0.032 | -86.60 | 0.192 | 1.90 | 3.27 |
| 1000.00 | 0.101 | -112.50 | 4.781 | -152.00 | 0.031 | -94.90 | 0.228 | 0.10 | 3.29 |
| 1100.00 | 0.090 | -108.50 | 4.843 | -167.90 | 0.031 | -103.90 | 0.256 | -0.60 | 3.15 |
| 1200.00 | 0.060 | -95.60 | 4.945 | 175.80 | 0.029 | -111.00 | 0.290 | -1.10 | 3.24 |
| 1300.00 | 0.019 | -79.20 | 4.999 | 159.50 | 0.029 | -120.20 | 0.308 | -0.30 | 3.16 |
| 1400.00 | 0.023 | 54.80 | 5.062 | 143.00 | 0.028 | -128.90 | 0.322 | -1.40 | 3.18 |
| 1500.00 | 0.062 | 80.70 | 5.114 | 126.40 | 0.029 | -133.10 | 0.327 | -2.20 | 3.08 |
| 1600.00 | 0.087 | 80.40 | 5.142 | 109.50 | 0.029 | -140.90 | 0.333 | -4.80 | 3.07 |
| 1700.00 | 0.113 | 78.70 | 5.160 | 92.70 | 0.029 | -146.20 | 0.344 | -7.00 | 3.02 |
| 1800.00 | 0.126 | 72.00 | 5.146 | 75.40 | 0.030 | -151.40 | 0.356 | -9.70 | 2.88 |
| 1900.00 | 0.154 | 63.50 | 5.123 | 58.00 | 0.032 | -159.70 | 0.371 | -11.10 | 2.70 |
| 2000.00 | 0.178 | 59.00 | 5.113 | 41.30 | 0.035 | -168.30 | 0.378 | -12.00 | 2.51 |
| 2100.00 | 0.212 | 54.20 | 5.063 | 24.00 | 0.036 | -175.70 | 0.383 | -12.80 | 2.39 |
| 2200.00 | 0.232 | 55.20 | 5.006 | 6.90 | 0.038 | 175.20 | 0.378 | -13.60 | 2.27 |
| 2300.00 | 0.246 | 53.80 | 4.954 | -10.40 | 0.041 | 165.20 | 0.367 | -16.10 | 2.13 |
| 2400.00 | 0.248 | 53.60 | 4.865 | -27.70 | 0.045 | 155.30 | 0.359 | -18.00 | 1.99 |
| 2500.00 | 0.240 | 49.20 | 4.783 | -45.00 | 0.048 | 143.60 | 0.356 | -21.10 | 1.88 |
| 2600.00 | 0.238 | 43.70 | 4.664 | -62.30 | 0.049 | 131.20 | 0.359 | -23.60 | 1.85 |
| 2700.00 | 0.240 | 36.20 | 4.529 | -79.60 | 0.052 | 119.80 | 0.366 | -26.20 | 1.76 |
| 2800.00 | 0.262 | 31.70 | 4.384 | -96.60 | 0.054 | 108.70 | 0.374 | -28.60 | 1.72 |
| 2900.00 | 0.285 | 28.80 | 4.255 | -113.10 | 0.056 | 95.50 | 0.372 | -31.10 | 1.68 |
| 3000.00 | 0.316 | 29.70 | 4.117 | -129.60 | 0.057 | 83.60 | 0.361 | -35.00 | 1.69 |

TYPICAL SCATTERING PARAMETERS (TA = 25C)

## UPC2712TB

VCC = 5 V, ICC = 13.9 mA

FREQUENCY	S11	S21	S12	S22	K

| (MHz) 100.00 | MAG 0.303 | ANG -8.10 | MAG 8.864 | ANG -16.70 | MAG 0.023 | ANG -11.40 | MAG 0.043 | ANG 2.30 | 2.32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200.00 | 0.291 | -10.10 | 8.827 | -33.50 | 0.023 | -19.20 | 0.055 | 11.50 | 2.35 |
| 300.00 | 0.295 | -11.80 | 8.936 | -49.50 | 0.022 | -25.50 | 0.078 | 8.50 | 2.38 |
| 400.00 | 0.276 | -11.30 | 9.044 | -67.60 | 0.023 | -34.60 | 0.095 | 13.40 | 2.33 |
| 500.00 | 0.265 | -11.00 | 9.051 | -82.20 | 0.023 | -42.80 | 0.112 | 13.60 | 2.37 |
| 600.00 | 0.243 | -12.30 | 9.096 | -98.80 | 0.023 | -50.00 | 0.120 | 11.10 | 2.35 |
| 700.00 | 0.222 | -20.30 | 9.089 | -115.20 | 0.023 | -59.80 | 0.120 | 1.70 | 2.37 |
| 800.00 | 0.219 | -25.40 | 9.080 | -131.50 | 0.023 | -66.20 | 0.136 | -6.00 | 2.38 |
| 900.00 | 0.230 | -33.90 | 9.096 | -147.60 | 0.023 | -73.00 | 0.155 | -14.40 | 2.39 |
| 1000.00 | 0.267 | -35.50 | 9.044 | -164.20 | 0.024 | -82.90 | 0.189 | -17.50 | 2.26 |
| 1100.00 | 0.290 | -35.50 | 9.197 | 179.50 | 0.024 | -89.50 | 0.212 | -19.90 | 2.12 |
| 1200.00 | 0.316 | -33.20 | 9.421 | 162.40 | 0.024 | -98.40 | 0.240 | -21.40 | 2.02 |
| 1300.00 | 0.317 | -30.60 | 9.524 | 144.90 | 0.024 | -107.00 | 0.245 | -23.20 | 1.94 |
| 1400.00 | 0.314 | -29.40 | 9.512 | 126.60 | 0.026 | -115.70 | 0.248 | -27.10 | 1.82 |
| 1500.00 | 0.296 | -28.10 | 9.574 | 109.10 | 0.026 | -122.30 | 0.236 | -31.80 | 1.78 |
| 1600.00 | 0.290 | -29.40 | 9.598 | 91.10 | 0.027 | -133.20 | 0.231 | -38.00 | 1.74 |
| 1700.00 | 0.278 | -31.10 | 9.480 | 72.90 | 0.028 | -139.40 | 0.221 | -43.80 | 1.72 |
| 1800.00 | 0.282 | -34.90 | 9.372 | 54.30 | 0.029 | -148.10 | 0.215 | -49.80 | 1.69 |
| 1900.00 | 0.284 | -35.50 | 9.193 | 35.60 | 0.030 | -157.60 | 0.199 | -53.00 | 1.70 |
| 2000.00 | 0.280 | -36.60 | 9.198 | 18.40 | 0.031 | -167.40 | 0.170 | -55.30 | 1.69 |
| 2100.00 | 0.273 | -36.00 | 9.011 | 0.10 | 0.033 | -175.10 | 0.134 | -56.20 | 1.68 |
| 2200.00 | 0.244 | -38.20 | 8.784 | -17.90 | 0.033 | 176.50 | 0.090 | -55.20 | 1.74 |
| 2300.00 | 0.222 | -40.00 | 8.717 | -35.10 | 0.034 | 164.80 | 0.050 | -53.70 | 1.74 |
| 2400.00 | 0.189 | -45.70 | 8.388 | -52.90 | 0.036 | 154.80 | 0.025 | 1.80 | 1.75 |
| 2500.00 | 0.177 | -52.90 | 8.217 | -70.10 | 0.037 | 143.50 | 0.039 | 33.40 | 1.74 |
| 2600.00 | 0.164 | -57.40 | 7.890 | -87.40 | 0.039 | 133.30 | 0.071 | 39.30 | 1.72 |
| 2700.00 | 0.158 | -59.60 | 7.597 | -104.60 | 0.041 | 123.80 | 0.099 | 34.30 | 1.70 |
| 2800.00 | 0.143 | -53.90 | 7.313 | -121.40 | 0.041 | 114.00 | 0.131 | 26.00 | 1.72 |
| 2900.00 | 0.128 | -44.30 | 7.078 | -138.40 | 0.043 | 101.40 | 0.149 | 22.80 | 1.70 |
| 3000.00 | 0.111 | -22.20 | 6.086 | -154.90 | 0.046 | 90.20 | 0.157 | 19.40 | 1.70 |

# UPC2711TB, UPC2712TB

OUTLINE DIMENSIONS (Units in mm)

#### PACKAGE OUTLINE S06

PIN CONNECTIONS

(Top View)	(Bottom View)

|  | C1G |  |
| --- | --- | --- |
| 3 |  | 4 |
| 2 |  | 5 |
| 1 |  | 6 |

| 4 |  | 3 |
| --- | --- | --- |
| 5 |  | 2 |
| 6 |  | 1 |

2.10.1

1.250.1

2.00.2

0.65  3

0.2 +0.1

C1G

-0

4

1.3

0.65  2	5

1	6	Marking is an example of UPC2711TB

0.9 0.1

0.7

0 ~0.1

DOT ON BACK SIDE

+0.1

0.15 -0.5

- Input

- GND

- GND

- Output

- GND

- VCC

TEST CIRCUIT

ORDERING INFORMATION (Solder Contains Lead)

| PART NUMBER | MARKING | QTY |
| --- | --- | --- |
| UPC2711TB-E3 | C1G | 3K/reel |
| UPC2712TB-E3 | C1H | 3K/reel |

VCC

1000 pF

6

1

4

1000 pF

2, 3, 5	1000 pF

Note:  Embossed tape, 8 mm wide. Pins 1, 2, and 3 face perforated side of tape.

50 Ω

50 Ω

## ORDERING INFORMATION (Pb-Free)

IN	OUT

| PART NUMBER | MARKING | QTY |
| --- | --- | --- |
| UPC2711TB-E3-A | C1G | 3K/reel |
| UPC2712TB-E3-A | C1H | 3K/reel |

Note:  Embossed tape, 8 mm wide. Pins 1, 2, and 3 face perforated side of tape.

Mouser Electronics

Authorized Distributor

Click to View Pricing, Inventory, Delivery & Lifecycle Information:

CEL:

### UPC2712TB-EVAL  UPC2711TB-EVAL  UPB1513TU-EVAL